Winning by Faithful Demonstration: the At-Fault Collision Tail is Controller-Limited, not Perception-Limited

An eight-probe study of why the nuPlan closed-loop metric saturates and why its residual collisions resist camera perception

Team NaLa  ·  Rank 1, nuPlan track  ·  Technical Report

# Abstract

We report a rank-1 submission to the nuPlan track of the AlpaSim E2E Closed-Loop Challenge 2026 built from a weightless, geometry-only route-follower (99 MB image, zero neural weights), and use it as an instrument to characterise the challenge's scoring surface. We establish three findings. First, the capability metric saturates: the top three teams cluster within 0.2% at ~1750, which we explain from first principles — fixed-duration rollouts and curvature-limited safe speed bound achievable distance to the recorded human demonstration, so any competent trajectory-tracking policy converges to the same near-optimal point. Second, through eight offline perception probes spanning six method-families — free-space, looming, object detection, tracked bounding-box TTC, monocular metric-depth TTC, and a full eight-camera surround early-warning survey — we show that none can separate the two at-fault collisions from ordinary safe car-following; indeed both collisions sit at the floor of the surround early-warning distribution. Third, we argue this is not a tuning failure but a structural property of the task: the obstacle is plainly visible in the forward camera in both failures, so the residual errors are controller-limited, not perception-limited. We conclude that the productive lever is longitudinal control faithfulness, not heavier perception, and that large multi-camera VLA models (e.g. Alpamayo) neither fit the 16 GiB submission budget nor address the identified bottleneck.

# 1. Problem setting

Task. Each submission serves an EgodriverService over gRPC, evaluated in closed loop on neural re-renderings of nuPlan scenes. Per rollout the driver receives eight cameras (CAM_F0, L0–L2, R0–R2, B0) at 1920×1080 JPEG at 2 Hz, its own estimated ego state, and a route as rig-frame waypoints. Constraints: mean Drive RPC ≤ 0.1 s, ≤ 16 GiB VRAM, image ≤ 40 GiB, no network, read-only rootfs.

Scoring. A zero-inflated ordinal-beta capability score via item-response theory, tiebroken by avg_dist_between_incidents_at_fault. Only front and lateral collisions are at-fault; rear collisions are free. Progress is gated on zero at-fault collisions and zero off-road.

Observation-space facts we verified. Tracing the protobuf schema and decoding saved rollout logs: the renderer_data side-channel is empty at evaluation; ground-truth actor poses are logged for scoring but not delivered to the driver; therefore the only obstacle signal available is the eight RGB cameras.

# 2. A weightless route-follower reaches rank 1

Our policy tracks the route centreline with a curvature-limited speed profile (v ≤ √(a_lat/κ)), a lateral re-centering term, and a safe decelerating fallback. It carries no learned weights, passed the throughput budget (2157 s of 2485 s), and scored a raw 0.980.

Figure 1. The public leaderboard saturates: the top three — two of them independent route-followers — cluster at ~1750. go_straight (1000) and mxr (below the floor after four submissions) bracket the achievable range.

The live competition is the collision tiebreak. With capability saturated, rank is decided by at-fault distance. We lead at 3.36; Cothlory reaches 3.89 at marginally lower capability. The practical question: can we raise at-fault distance without regressing the scenes we already pass?

| Rank / Team | Capability score | Dist. btwn at-fault | Dist. to GT |
| --- | --- | --- | --- |
| 1 NaLa (ours) | 1750.56 | 3.36 | 0.98 |
| 2 metamon | 1750.56 | 3.09 | 2.65 |
| 3 Cothlory | 1746.98 | 3.89 | 0.56 |
| 4 Host policy1 | 1600 | 1.68 | 3.07 |
| 6 Host go_straight | 1000 | 0.28 | 2.47 |
| 7-10 mxr (×4) | 846-853 | ~1.0 | 0.23 |

Table 1. nuPlan leaderboard at time of submission.

# 3. The collision tail: two failures, one shape

On a 200-scene local replay our policy records mean ~0.99 with exactly two at-fault collisions and zero off-road. Rendered to video and diagnosed frame-by-frame:

- Crossing (scene …8cc8fe63): a vehicle enters from the right into the forward path; ~2.0 s of warning, visible in CAM_F0 before impact.

- Rear-end (scene …dd162898): stop-and-go traffic; the lead vehicle halts, ego does not. ~2.2 s of warning, the lead visible dead-ahead the entire clip.

Both share one shape: a forward obstacle in the path, ~2 s of warning, and a policy that never consults a camera. The obstacle is visible; the warning budget is comfortable. On its face a textbook forward-collision-braking problem — so we tried to build the brake.

# 4. Eight probes, six method-families, one negative result

Every candidate was evaluated identically: compute a per-scene danger signal offline on the saved frames, then ask whether a single threshold separates the two collision scenes from the 198 clear ones. This gate costs no submissions and no simulation, and it makes the regression risk explicit — a signal that also fires on many clear scenes would brake needlessly and collapse progress, the failure visible in the mxr entries.

Figure 2. For each method-family, clear scenes (of 198) that must be false-flagged to catch both collisions. A usable trigger sits at or below the dashed line (≤5). None does; free-space never fires and surround early-warning inverts.

| Method | Signal & why it fails | Clear FP /198 |
| --- | --- | --- |
| 1. Free-space | Vertical-gradient edge → pinhole range. Never fired on the neural renders. | never fires |
| 2. Looming | Lower-centre blob-area growth. Confounded by scene brightness/clutter. | 43 |
| 3. Static detection | YOLOv8 threat = area×centrality×verticality. Safe lead-cars look identical; one peak was a pedestrian. | 16 |
| 4. bbox-height TTC | YOLO+ByteTrack height→closing→TTC. Signal inverts: 74 clear scenes had lower TTC than collisions. | 74 |
| 5. Depth-metric TTC | Depth-Anything-V2 range at tracked box → metric TTC. Depth did not rescue; collision ranked 3rd. | ≥60 |
| 6-8. Surround early-warning | All 8 cameras. Do R0/L0 see a forward-path threat before F0? Both collisions sit at the FLOOR (0.0, 0.06) vs clear median 0.14. | 198 (inverts) |

Table 2. The six method-families and their separation failure.

The decisive probe: surround early-warning. Because forward-only cues failed, we tested the natural hypothesis that the forward-side cameras (R0/L0) see a crossing threat before CAM_F0. We ran YOLO across all eight cameras on all 200 scenes and scored, per scene, the peak forward-path threat visible in R0/L0 while F0 still saw nothing. The hypothesis is not merely unsupported — it is inverted: both collisions sit at the floor of the distribution (early-warning 0.00 and 0.06) against a clear-scene median of 0.14 and max of 0.94. Every one of the 198 clear scenes has early-warning at least as high as the worst collision.

Figure 3. Surround early-warning across 200 scenes (clear distribution illustrative, matched to measured median 0.14 / max 0.94). Both at-fault collisions (red) sit at the floor — the side cameras carry least warning exactly where it is needed.

| Camera | Mean peak threat | Mean hit-frames | Role in policy |
| --- | --- | --- | --- |
| CAM_F0 | 0.052 | 3.40 | forward — braking cue |
| CAM_R0 | 0.125 | 2.25 | fwd-side — early-warn (tested) |
| CAM_L0 | 0.148 | 3.65 | fwd-side — early-warn (tested) |
| CAM_R1/R2 | 0.13–0.18 | 2–3 | rear-side — ignored (rear not at-fault) |
| CAM_L1/L2 | 0.12–0.17 | 3.6–3.9 | rear-side — ignored |
| CAM_B0 | 0.050 | 3.92 | rear — ignored |

Table 3. Per-camera threat statistics (means over 200 scenes). High side-camera activity correlates with busy intersections the policy navigates safely, not with the collisions.

Why they converge on failure. The methods span appearance, semantics, temporal geometry, metric depth, and multi-view coverage, yet all fail for one reason: the visual situation immediately preceding an at-fault collision is nearly identical to the many safe situations the policy handles correctly — following a lead car, approaching a junction, yielding to cross-traffic. The collisions happen with the obstacle dead ahead in CAM_F0; by the time it matters it is in front, which is exactly why side cameras carry no early warning and why high side-camera activity marks safe busy scenes instead. At 2 Hz, from any camera, “safely following” and “about to hit” are not separable.

# 5. The distance lever is also closed

If collisions cannot be safely cut, the metric distance ÷ incidents might be raised by driving more distance. We measured ego distance across all 200 rollouts.

Figure 4. Per-scene mean ego speed (illustrative, matched to measured quantiles: mean 6.66, median 6.89, max 15.3 m/s). Speed sits far below the 20 m/s cap because urban geometry and traffic limit it, not conservatism.

Every clip is exactly 4.52 s, so distance is speed × constant. Mean ego speed is 6.66 m/s against a 20 m/s cap, with 125 of 200 scenes under 8 m/s. This looks like slack but is not: the low speed is imposed by curvature and traffic, and our distance-to-ground-truth of 0.98 shows the policy already reproduces the recorded human trajectory. Driving further means diverging from the safe demonstration into off-road and collision — trading wins for zeros.

# 6. Interpretation: controller-limited, not perception-limited

The eight probes converge on a conclusion sharper than “perception is hard here.” In both failures the obstacle is plainly and continuously visible in the forward camera; what is missing is not detection but response. A blind route-follower reproduces the demonstrated trajectory faithfully — which is why it wins — but has no mechanism to deviate when the closing geometry exceeds what the demonstration assumed. The residual tail is therefore a control problem: the policy sees the lead car and the crosser and simply does not yield or brake.

Why heavier perception does not help. This directly bounds the value of large multi-camera VLA policies. NVIDIA's Alpamayo family consumes the same cameras and, by its own documentation, is expected to struggle to see cross-traffic in a right turn from a forward view — the exact structure of our crossing collision. It also cannot be deployed here: inference requires ≥ 24 GiB VRAM (OOM at 16 GiB) with 22 GB weights under a non-commercial license, against our 16 GiB budget. Bigger perception aimed at a control-limited tail is effort mis-spent.

# 7. The productive lever: longitudinal-control faithfulness

[Phase 2 — in progress.]

The evidence redirects effort from seeing to reacting. The one lever untouched by the perception ceiling is the controller's longitudinal behaviour: a demonstration-faithful policy that additionally yields the minimal amount needed when the forward corridor closes — keyed on the very CAM_F0 signal that is always present — could address the rear-end failure (a pure response gap) without any new perception. The design constraint is set by our own ablations: any intervention must be idle when the corridor is clear, so the 198 passing scenes are untouched by construction, avoiding the mxr regression. We will evaluate candidate longitudinal governors against the identical offline separation gate before spending any submission.

- Hypothesis: a CAM_F0-keyed longitudinal yield, active only when a centred forward threat is both large and growing, separates the rear-end collision from safe following at low false-positive cost — where every fixed-threshold perception cue failed.

- Guardrail: regression-safety by construction — the governor is a no-op on clear corridors, preserving the demonstration-matched trajectory that earns rank 1.

# 8. Contributions

- A first-principles account of the metric's saturation (fixed-duration × curvature-limited safe speed × demonstration-matched distance).

- A rigorous negative result: eight probes across six perception method-families, on an identical offline gate, none able to isolate the at-fault tail — with the mechanistic explanation that the collisions are visually identical to safe driving.

- The controller-limited thesis: evidence that the residual failures are response gaps, not detection gaps, bounding the value of heavier perception (including large VLA models that also violate the compute budget).

- A reproducible offline methodology on saved rollout logs — no simulation, no submissions — enabling honest, pre-registered iteration.

Figures 1–2 use measured values; the clear-scene distributions in Figures 3–4 are illustrative, matched to measured quantiles. Section 7 (Phase 2) in progress. Team NaLa, nuPlan track.